Fig. Instrument arrangement for recording process parameters at the TCM site of strip steel.

List of process variables for strip TCR manufacturing industry

| ID | Variable | Physical Description | Unit | Notes |
| --- | --- | --- | --- | --- |
| 1 | TN\L2_TRK_LEN_FRM_S1 | Distance from weld seam to STD#1 |  |  |
| 2 | TN\L2_TRK_LEN_FRM_S2 | Distance from weld seam to STD#2 |  |  |
| 3 | TN\L2_TRK_LEN_FRM_S3 | Distance from weld seam to STD#3 |  |  |
| 4 | TN\L2_TRK_LEN_FRM_S4 | Distance from weld seam to STD#4 |  |  |
| 5 | TN\L2_TRK_LEN_FRM_S5 | Distance from weld seam to STD#5 |  |  |
| 6 | TN\L2_TRK_LEN_FRM_V1 | Distance from weld seam to the thickness gauge at the entrance of STD#1 |  |  |
| 7 | TN\L2_TRK_LEN_FRM_V2 | Distance from weld seam to the thickness gauge at the exit of STD#1 |  |  |
| 8 | TN\L2_TRK_LEN_FRM_V5 | Distance from weld seam to the thickness gauge at the exit of STD#4 |  |  |
| 9 | TN\L2_TRK_LEN_FRM_H5 | Distance from weld seam to the thickness gauge at the exit ofSTD#5 |  |  |
| 10 | TN\L2_TRK_LEN_FRM_ED | Distance from weld seam to the edge drop gauge |  |  |
| 11 | TN\L2_TRK_LEN_FRM_SP | Distance from weld seam to the shape meter |  |  |
| 12 | TN\L2_AGC_V1THK_ACT | Thickness at the entrance of STD#1 | mm |  |
| 13 | TN\L2_AGC_V2THK_ACT | Thickness at the exit of STD#1 | mm | Target Predictive Variables THK_STD#1 |
| 14 | TN\L2_AGC_V5THK_ACT | Thickness at the exit of STD#4 | mm | Target Predictive Variables THK_STD#4 |
| 15 | TN\L2_AGC_H5THK_ACT | Thickness at the exit of STD#5 | mm | Target Predictive Variables THK_STD#5 |
| 16 | TN\L2_TCM_BR8_SPD_ACT | Roller speed of tension roller#8 | mpm |  |
| 17 | TN\L2_TCM_S1_SPD_ACT | Roller speed of STD#1 | mpm |  |
| 18 | TN\L2_TCM_S2_SPD_ACT | Roller speed of STD#2 | mpm |  |
| 19 | TN\L2_TCM_S3_SPD_ACT | Roller speed of STD#3 | mpm |  |
| 20 | TN\L2_TCM_S4_SPD_ACT | Roller speed of STD#4 | mpm |  |
| 21 | TN\L2_TCM_S5_SPD_ACT | Roller speed of STD#5 | mpm |  |
| 22 | TN\L2_AGC_V1SPD_ACT | Strip speed at the entrance of STD#1 | mpm |  |
| 23 | TN\L2_AGC_V2SPD_ACT | Strip speed at the exit of STD#1 | mpm |  |
| 24 | TN\L2_AGC_V5SPD_ACT | Strip speed at the exit of STD#4 | mpm |  |
| 25 | TN\L2_AGC_H5SPD_ACT | Strip speed at the exit of STD#5 | mpm |  |
| 26 | TN\L2_TCM_S1_FWD_SLIP | Forward slip of STD#1 |  |  |
| 27 | TN\L2_TCM_S2_FWD_SLIP | Forward slip of STD#2 |  |  |
| 28 | TN\L2_TCM_S3_FWD_SLIP | Forward slip of STD#3 |  |  |
| 29 | TN\L2_TCM_S4_FWD_SLIP | Forward slip of STD#4 |  |  |
| 30 | TN\L2_TCM_S5_FWD_SLIP | Forward slip of STD#5 |  |  |
| 31 | TN\L2_AGC_S1TRFC_ACT | Rolling force of STD#1 (Hydraulic head sensor) | kN | Target Predictive Variables RFC_STD#1 |
| 32 | TN\L2_AGC_S2TRFC_ACT | Rolling force of STD#2 (Hydraulic head sensor) | kN | Target Predictive Variables RFC_STD#2 |
| 33 | TN\L2_AGC_S3TRFC_ACT | Rolling force of STD#3 (Hydraulic head sensor) | kN | Target Predictive Variables RFC_STD#3 |
| 34 | TN\L2_AGC_S4TRFC_ACT | Rolling force of STD#4 (Hydraulic head sensor) | kN | Target Predictive Variables RFC_STD#4 |
| 35 | TN\L2_AGC_S5TRFC_ACT | Rolling force of STD#5 (Hydraulic head sensor) | kN | Target Predictive Variables RFC_STD#5 |
| 36 | TN\L2_AGC_S1TLC_ACT | Rolling force of STD#1 (Roller surface sensor) | kN |  |
| 37 | TN\L2_AGC_S5TLC_ACT | Rolling force of STD#5 (Roller surface sensor) | kN |  |
| 38 | TN\L2_AGC_S1DRFC_ACT | Deviation between actual and calculated rolling force of STD#1 (Hydraulic head sensor) |  |  |
| 39 | TN\L2_AGC_S2DRFC_ACT | Deviation between actual and calculated rolling force of STD#2 (Hydraulic head sensor) |  |  |
| 40 | TN\L2_AGC_S3DRFC_ACT | Deviation between actual and calculated rolling force of STD#3 (Hydraulic head sensor) |  |  |
| 41 | TN\L2_AGC_S4DRFC_ACT | Deviation between actual and calculated rolling force of STD#4 (Hydraulic head sensor) |  |  |
| 42 | TN\L2_AGC_S5DRFC_ACT | Deviation between actual and calculated rolling force of STD#5 (Hydraulic head sensor) |  |  |
| 43 | TN\L2_AGC_S1DLC_ACT | Deviation between actual and calculated rolling force of STD#1 (Roller surface sensor) |  |  |
| 44 | TN\L2_AGC_S5DLC_ACT | Deviation between actual and calculated rolling force of STD#5 (Roller surface sensor) |  |  |
| 45 | TN\L2_TCM_ENTUTEN_ACT | Unit tension at the entrance of the rolling mill | kN/mm2 |  |
| 46 | TN\L2_AGC_S12UTEN_ACT | Unit tension at the exit of STD#1 | kN/mm2 |  |
| 47 | TN\L2_AGC_S23UTEN_ACT | Unit tension at the exit of STD#2 | kN/mm2 |  |
| 48 | TN\L2_AGC_S34UTEN_ACT | Unit tension at the exit of STD#3 | kN/mm2 |  |
| 49 | TN\L2_AGC_S45UTEN_ACT | Unit tension at the exit of STD#4 | kN/mm2 |  |
| 50 | TN\L2_TCM_DELUTEN_ACT | Unit tension at the exit of STD#5 | kN/mm2 |  |
| 51 | TN\L2_TCM_ENTTTEN_ACT | Total tension at the entrance of the rolling mill | kN |  |
| 52 | TN\L2_AGC_S12TTEN_ACT | Total tension at the exit of STD#1 | kN |  |
| 53 | TN\L2_AGC_S23TTEN_ACT | Total tension at the exit of STD#2 | kN |  |
| 54 | TN\L2_AGC_S34TTEN_ACT | Total tension at the exit of STD#3 | kN |  |
| 55 | TN\L2_AGC_S45TTEN_ACT | Total tension at the exit of STD#4 | kN |  |
| 56 | TN\L2_TCM_DELTTEN_ACT | Total tension at the exit of STD#5 | kN |  |
| 57 | TN\L2_TCM_ENTDUTEN_ACT | Deviation between actual and calculated unit tension at the entrance of the rolling mill | kN/mm |  |
| 58 | TN\L2_AGC_S12DUTEN_ACT | Deviation between actual and calculated unit tension at the exit of STD#1 |  |  |
| 59 | TN\L2_AGC_S23DUTEN_ACT | Deviation between actual and calculated unit tension at the exit of STD#2 |  |  |
| 60 | TN\L2_AGC_S34DUTEN_ACT | Deviation between actual and calculated unit tension at the exit of STD#3 |  |  |
| 61 | TN\L2_AGC_S45DUTEN_ACT | Deviation between actual and calculated unit tension at the exit of STD#4 |  |  |
| 62 | TN\L2_TCM_DELDUTEN_ACT | Deviation between actual and calculated unit tension at the exit of STD#5 |  |  |
| 63 | TN\L2_TCM_ENTDTTEN_ACT | Deviation between actual and calculated tension at the entrance of the rolling mill |  |  |
| 64 | TN\L2_AGC_S12DTTEN_ACT | Deviation between actual and calculated total tension at the exit of STD#1 |  |  |
| 65 | TN\L2_AGC_S23DTTEN_ACT | Deviation between actual and calculated total tension at the exit of STD#2 |  |  |
| 66 | TN\L2_AGC_S34DTTEN_ACT | Deviation between actual and calculated total tension at the exit of STD#3 |  |  |
| 67 | TN\L2_AGC_S45DTTEN_ACT | Deviation between actual and calculated total tension at the exit of STD#4 |  |  |
| 68 | TN\L2_TCM_DELDTTEN_ACT | Deviation between actual and calculated total tension at the exit of STD#5 |  |  |
| 69 | TN\L2_TCM_BR7_1TQ_ACT | Torque of Tension Roller#7, Unit 1 |  |  |
| 70 | TN\L2_TCM_BR7_2TQ_ACT | Torque of Tension Roller#7, Unit 2 |  |  |
| 71 | TN\L2_TCM_BR8_1TQ_ACT | Torque of Tension Roller#8, Unit 1 |  |  |
| 72 | TN\L2_TCM_BR8_2TQ_ACT | Torque of Tension Roller#8, Unit 2 |  |  |
| 73 | TN\L2_TCM_S1TQ_ACT | Motor torque of STD#1 |  |  |
| 74 | TN\L2_TCM_S2TQ_ACT | Motor torque of STD#2 |  |  |
| 75 | TN\L2_TCM_S3TQ_ACT | Motor torque of STD#3 |  |  |
| 76 | TN\L2_TCM_S4TQ_ACT | Motor torque of STD#4 |  |  |
| 77 | TN\L2_TCM_S5TQ_ACT | Motor torque of STD#5 |  |  |
| 78 | TN\L2_TCM_BR7_1PW_ACT | Motor power of Tension Roller#7, Unit 1 |  |  |
| 79 | TN\L2_TCM_BR7_2PW_ACT | Motor power of Tension Roller#7, Unit 2 |  |  |
| 80 | TN\L2_TCM_BR8_1PW_ACT | Motor power of Tension Roller#8, Unit 1 |  |  |
| 81 | TN\L2_TCM_BR8_2PW_ACT | Motor power of Tension Roller#8, Unit 2 |  |  |
| 82 | TN\L2_TCM_S1PW_ACT | Motor power of STD#1 |  |  |
| 83 | TN\L2_TCM_S2PW_ACT | Motor power of STD#2 |  |  |
| 84 | TN\L2_TCM_S3PW_ACT | Motor power of STD#3 |  |  |
| 85 | TN\L2_TCM_S4PW_ACT | Motor power of STD#4 |  |  |
| 86 | TN\L2_TCM_S5PW_ACT | Motor power of STD#5 |  |  |
| 87 | TN\L2_AGC_S1GAP_ACT | Roll gap of STD#1 | mm |  |
| 88 | TN\L2_AGC_S2GAP_ACT | Roll gap of STD#2 | mm |  |
| 89 | TN\L2_AGC_S3GAP_ACT | Roll gap of STD#3 | mm |  |
| 90 | TN\L2_AGC_S4GAP_ACT | Roll gap of STD#4 | mm |  |
| 91 | TN\L2_AGC_S5GAP_ACT | Roll gap of STD#5 | mm |  |
| 92 | TN\L2_AGC_S1LEV_ACT | Leveling of STD#1 | mm |  |
| 93 | TN\L2_AGC_S2LEV_ACT | Leveling of STD#2 | mm |  |
| 94 | TN\L2_AGC_S3LEV_ACT | Leveling of STD#3 | mm |  |
| 95 | TN\L2_AGC_S4LEV_ACT | Leveling of STD#4 | mm |  |
| 96 | TN\L2_AGC_S5LEV_ACT | Leveling of STD#5 | mm |  |
| 97 | TN\L2_AFC_S1WRB_ACT | Work roll bending force of STD#1 | kN/side |  |
| 98 | TN\L2_AFC_S2WRB_ACT | Work roll bending force of STD#2 | kN/side |  |
| 99 | TN\L2_AFC_S3WRB_ACT | Work roll bending force of STD#3 | kN/side |  |
| 100 | TN\L2_AFC_S4WRB_ACT | Work roll bending force of STD#4 | kN/side |  |
| 101 | TN\L2_AFC_S5WRB_ACT | Work roll bending force of STD#5 | kN/side |  |
| 102 | TN\L2_AFC_S1IRB_ACT | Intermediate roll bending force of STD#1 | kN/side |  |
| 103 | TN\L2_AFC_S2IRB_ACT | Intermediate roll bending force of STD#2 | kN/side |  |
| 104 | TN\L2_AFC_S3IRB_ACT | Intermediate roll bending force of STD#3 | kN/side |  |
| 105 | TN\L2_AFC_S4IRB_ACT | Intermediate roll bending force of STD#4 | kN/side |  |
| 106 | TN\L2_AFC_S5IRB_ACT | Intermediate roll bending force of STD#5 | kN/side |  |
| 107 | TN\L2_AFC_S1TIRS_ACT | Top intermediate roll shifting of STD#1 | mm |  |
| 108 | TN\L2_AFC_S1BIRS_ACT | Bottom intermediate roll shifting of STD#1 | mm |  |
| 109 | TN\L2_AFC_S2TIRS_ACT | Top intermediate roll shifting of STD#2 | mm |  |
| 110 | TN\L2_AFC_S2BIRS_ACT | Bottom intermediate roll shifting of STD#2 | mm |  |
| 111 | TN\L2_AFC_S3TIRS_ACT | Top intermediate roll shifting of STD#3 | mm |  |
| 112 | TN\L2_AFC_S3BIRS_ACT | Bottom intermediate roll shifting of STD#3 | mm |  |
| 113 | TN\L2_AFC_S4TIRS_ACT | Top intermediate roll shifting of STD#4 | mm |  |
| 114 | TN\L2_AFC_S4BIRS_ACT | Bottom intermediate roll shifting of STD#4 | mm |  |
| 115 | TN\L2_AFC_S5TIRS_ACT | Top intermediate roll shifting of STD#5 | mm |  |
| 116 | TN\L2_AFC_S5BIRS_ACT | Bottom intermediate roll shifting of STD#5 | mm |  |
| 117 | TN\L2_TCM_COOL_FL_S1 | Emulsion flow rate of STD#1 | l/min |  |
| 118 | TN\L2_TCM_COOL_FL_S2 | Emulsion flow rate of STD#2 | l/min |  |
| 119 | TN\L2_TCM_COOL_FL_S3 | Emulsion flow rate of STD#3 | l/min |  |
| 120 | TN\L2_TCM_COOL_FL_S4 | Emulsion flow rate of STD#4 | l/min |  |
| 121 | TN\L2_TCM_COOL_FL_S5 | Emulsion flow rate of STD#5 | l/min |  |
| 122 | TN\L2_TCM_TR1TQ_ACT | Motor torque of winder#1 |  |  |
| 123 | TN\L2_TCM_TR1PW_ACT | Motor power of winder#1 |  |  |
| 124 | TN\G211_122NI | Total tension of the inline accumulator (BR7-BR8) |  |  |
| 125 | TN\G301_347NI | Position of the inline accumulator (BR7-BR8) |  |  |
| 126 | TN\L2_AFC_EGDWS10_ACT | Deviation in thickness between the center and the edge of the strip, 10 mm from the operation side |  |  |
| 127 | TN\L2_AFC_EGDWS15_ACT | Deviation in thickness between the center and the edge of the strip, 15 mm from the operation side |  |  |
| 128 | TN\L2_AFC_EGDWS20_ACT | Deviation in thickness between the center and the edge of the strip, 20 mm from the operation side |  |  |
| 129 | TN\L2_AFC_EGDWS25_ACT | Deviation in thickness between the center and the edge of the strip, 25 mm from the operation side |  |  |
| 130 | TN\L2_AFC_EGDWS50_ACT | Deviation in thickness between the center and the edge of the strip, 50 mm from the operation side |  |  |
| 131 | TN\L2_AFC_EGDWS75_ACT | Deviation in thickness between the center and the edge of the strip, 75 mm from the operation side |  |  |
| 132 | TN\L2_AFC_EGDWS100_ACT | Deviation in thickness between the center and the edge of the strip, 100 mm from the operation side |  |  |
| 133 | TN\L2_AFC_EGDWS115_ACT | Deviation in thickness between the center and the edge of the strip, 115 mm from the operation side |  |  |
| 134 | TN\L2_AFC_EGDDS10_ACT | Deviation in thickness between the center and the edge of the strip, 10 mm from the drive side |  |  |
| 135 | TN\L2_AFC_EGDDS15_ACT | Deviation in thickness between the center and the edge of the strip, 15 mm from the drive side |  |  |
| 136 | TN\L2_AFC_EGDDS20_ACT | Deviation in thickness between the center and the edge of the strip, 20 mm from the drive side |  |  |
| 137 | TN\L2_AFC_EGDDS25_ACT | Deviation in thickness between the center and the edge of the strip, 25 mm from the drive side |  |  |
| 138 | TN\L2_AFC_EGDDS50_ACT | Deviation in thickness between the center and the edge of the strip, 50 mm from the drive side |  |  |
| 139 | TN\L2_AFC_EGDDS75_ACT | Deviation in thickness between the center and the edge of the strip, 75 mm from the drive side |  |  |
| 140 | TN\L2_AFC_EGDDS100_ACT | Deviation in thickness between the center and the edge of the strip, 100 mm from the drive side |  |  |
| 141 | TN\L2_AFC_EGDDS115_ACT | Deviation in thickness between the center and the edge of the strip, 115 mm from the drive side |  |  |
| 142 | TN\L2_AFC_H5WID_ACT | Strip width | mm |  |
| 143 | TN\L2_AFC_H5WID_OFST | Deviation between calculated and actual strip width |  |  |
| 144 | TN\L2_AFC_S1WRB_MAN | Manual adjustment of the work roll bending force in STD#1 |  |  |
| 145 | TN\L2_AFC_S2WRB_MAN | Manual adjustment of the work roll bending force in STD#2 |  |  |
| 146 | TN\L2_AFC_BAFFC_W_COR | Bending AFFC (Automatic Flatness Feedback Control) for work roll bending force correction |  |  |
| 147 | TN\L2_AFC_LAFFC_COR | Leveling AFFC (Automatic Flatness Feedback Control) for roll leveling correction |  |  |
| 148 | TN\L2_AFC_S1FFC_W_CMP | Force Following Control (FFC) compensation for work rolls in STD#1 |  |  |
| 149 | TN\L2_AFC_S2FFC_W_CMP | Force Following Control (FFC) compensation for work rolls in STD#2 |  |  |
| 150 | TN\L2_AFC_S3FFC_W_CMP | Force Following Control (FFC) compensation for work rolls in STD#3 |  |  |
| 151 | TN\L2_AFC_S4FFC_W_CMP | Force Following Control (FFC) compensation for work rolls in STD#4 |  |  |
| 152 | TN\L2_AFC_S5FFC_W_CMP | Force Following Control (FFC) compensation for work rolls in STD#5 |  |  |
| 153 | TN\L2_AFC_S1FFC_I_CMP | Force Following Control (FFC) compensation for intermediate rolls in STD#1 |  |  |
| 154 | TN\L2_AFC_S2FFC_I_CMP | Force Following Control (FFC) compensation for intermediate rolls in STD#2 |  |  |
| 155 | TN\L2_AFC_S3FFC_I_CMP | Force Following Control (FFC) compensation for intermediate rolls in STD#3 |  |  |
| 156 | TN\L2_AFC_S4FFC_I_CMP | Force Following Control (FFC) compensation for intermediate rolls in STD#4 |  |  |
| 157 | TN\L2_AFC_S5FFC_I_CMP | Force Following Control (FFC) compensation for intermediate rolls in STD#5 |  |  |
| 158 | TN\L2_AGC_V1THK_DEV | Deviation between the actual and calculated thickness at the entrance of STD#1 |  |  |
| 159 | TN\L2_AGC_V2THK_DEV | Deviation between the actual and calculated thickness at the exit of STD#1 |  |  |
| 160 | TN\L2_AGC_V5THK_DEV | Deviation between the actual and calculated thickness at the exit of STD#4 |  |  |
| 161 | TN\L2_AGC_H5THK_DEV | Deviation between the actual and calculated thickness at the exit of STD#5 |  |  |
| 162 | TN\L2_TCM_MRH | Main Reference Speed |  |  |
| 163 | TN\L2_CPC_DEV_10 | Deviation of CPC (Central Line Position Control) #10 |  |  |
| 164 | TN\L2_CPC_DEV_11 | Deviation of CPC (Central Line Position Control) #11 |  |  |
| 165 | TN\G111_60NI | Feedback from the hydraulic cylinder at the outlet of inline loop at CPC (Central Line Position Control) #10 |  |  |
| 166 | TN\G111_61NI | Feedback from the hydraulic cylinder at the outlet of CPC (Central Line Position Control) BR#11 |  |  |
| 167 | TN\L2_AFC_S1FFC_LKFC | Measured value of flatness feedforward control in STD#1 |  |  |
| 168 | TN\L2_AFC_S2FFC_LKFC | Measured value of flatness feedforward control in STD#2 |  |  |
| 169 | TN\L2_TCM_BITE_S5_A | Emulsion flow rate in region 12-A of STD#5 |  |  |
| 170 | TN\L2_TCM_BITE_S5_B | Emulsion flow rate in region 12-B of STD#5 |  |  |